ESP32 Industrial I/O Module

Bill of Materials & GPIO Pinout Reference

Rev 1.0  |  June 2026  |  Target MCU: ESP32-WROOM-32D

# 1. System Overview

This module is based on the ESP32-WROOM-32D and provides a complete industrial I/O interface including:

•  4× Momentary push buttons (GPIO, internal pull-up)

•  2× Analog inputs (ADC1, 0–3.3V or 0–10V with divider)

•  4× 12V tolerant digital inputs (optocoupler isolated)

•  4× PWM-capable outputs (LEDC peripheral, N-ch MOSFET driven)

•  I2C LCD display (PCF8574 backpack, 16×2 or 20×4)

•  CAN bus (TWAI peripheral + SN65HVD230 transceiver)

•  UART GPS module (UART2, remapped to GPIO 13/23)

•  SparkFun FTDI programming header (UART0, auto-reset circuit)

NOTE: All GPIO signals operate at 3.3V logic. External signals at 5V or 12V must be level-shifted or isolated before connecting to ESP32 pins. GPIO 34–39 are input-only and have no internal pull-up/down resistors.

# 2. ESP32 GPIO Pinout Diagram

The diagram below shows the functional assignment of all used GPIO pins on the ESP32-WROOM-32D module.

┌────────────────────────────────────┐

│          ESP32-WROOM-32             │

│                                     │

3V3         ──── 1│                                     │38 ──── GND

GND         ──── 2│                                     │37 ──── IO23  → GPS TX

IO34 ← BTN4 ──── 3│                                     │36 ──── IO22  → I2C SCL (LCD)

IO35 ← BTN3 ──── 4│                                     │35 ──── IO21  → I2C SDA (LCD)

IO32 ← BTN1 ──── 5│                                     │34 ──── IO19  → PWM OUT 4

IO33 ← BTN2 ──── 6│                                     │33 ──── IO18  → PWM OUT 3

IO25 ← 12V IN 1 - 7│                                     │32 ──── IO17  → PWM OUT 2

IO26 ← 12V IN 2 - 8│                                     │31 ──── IO16  → PWM OUT 1

IO27 ← 12V IN 3 - 9│                                     │30 ──── IO13  → GPS RX

IO14 ← 12V IN 4 -10│                                     │29 ──── IO5   → CAN RX

IO36 ← AIN 1   ─ 11│                                     │28 ──── IO4   → CAN TX

IO39 ← AIN 2   ─ 12│                                     │27 ──── IO3   → UART0 RX (FTDI)

13│                                     │26 ──── IO1   → UART0 TX (FTDI)

14│                                     │25 ──── IO0   → FTDI DTR/BOOT

│                                     │

│           USB / FTDI Header         │

└────────────────────────────────────┘

# 3. GPIO Pin Assignment Table

| GPIO | Function | Direction | Notes |
| --- | --- | --- | --- |
| GPIO 0 | BOOT / FTDI DTR | Input | Auto-reset via 100nF cap from FTDI DTR |
| GPIO 1 | UART0 TX | Output | FTDI RX — programming & debug |
| GPIO 3 | UART0 RX | Input | FTDI TX — programming & debug |
| GPIO 4 | CAN TX (TWAI) | Output | To SN65HVD230 TXD pin |
| GPIO 5 | CAN RX (TWAI) | Input | From SN65HVD230 RXD pin |
| GPIO 13 | UART2 RX (GPS) | Input | GPS module TX → ESP RX |
| GPIO 14 | 12V Digital IN 4 | Input | Via PC817 optocoupler |
| GPIO 16 | PWM OUT 1 (LEDC) | Output | Via IRLZ44N MOSFET + gate resistor |
| GPIO 17 | PWM OUT 2 (LEDC) | Output | Via IRLZ44N MOSFET + gate resistor |
| GPIO 18 | PWM OUT 3 (LEDC) | Output | Via IRLZ44N MOSFET + gate resistor |
| GPIO 19 | PWM OUT 4 (LEDC) | Output | Via IRLZ44N MOSFET + gate resistor |
| GPIO 21 | I2C SDA | Bidirectional | LCD PCF8574 backpack |
| GPIO 22 | I2C SCL | Output | LCD PCF8574 backpack |
| GPIO 23 | UART2 TX (GPS) | Output | GPS module RX ← ESP TX |
| GPIO 25 | 12V Digital IN 1 | Input | Via PC817 optocoupler |
| GPIO 26 | 12V Digital IN 2 | Input | Via PC817 optocoupler |
| GPIO 27 | 12V Digital IN 3 | Input | Via PC817 optocoupler |
| GPIO 32 | Button 1 | Input | Internal pull-up, active-low |
| GPIO 33 | Button 2 | Input | Internal pull-up, active-low |
| GPIO 34 | Button 3 (input-only) | Input | Input-only GPIO, no internal pull-up — add 10kΩ |
| GPIO 35 | Button 4 (input-only) | Input | Input-only GPIO, no internal pull-up — add 10kΩ |
| GPIO 36 (VP) | Analog IN 1 | Analog Input | ADC1_CH0 — 0–3.3V (use divider for higher) |
| GPIO 39 (VN) | Analog IN 2 | Analog Input | ADC1_CH3 — 0–3.3V (use divider for higher) |

# 4. Bill of Materials

All quantities are per one assembled board. Substitute components may be used provided electrical specifications are met.

| Ref | Qty | Description | Part / Value | Package | Notes |
| --- | --- | --- | --- | --- | --- |
| U1 | 1 | Microcontroller Module | ESP32-WROOM-32D | Module | 38-pin, 4MB flash, 3.3V |
| U2 | 1 | CAN Bus Transceiver | SN65HVD230DR | SOIC-8 | 3.3V, 1Mbps, ISO 11898 |
| U3 | 1 | FTDI USB-Serial Bridge | SparkFun FTDI Basic 3.3V | Module | Programming header |
| U4 | 1 | I2C LCD Backpack | PCF8574T + 16×2 LCD | Module | I2C addr 0x27 |
| U5 | 1 | 3.3V LDO Regulator | AMS1117-3.3 | SOT-223 | 800mA, 12V→3.3V |
| U6 | 4 | Optocoupler (12V DIG IN) | PC817C | DIP-4 | 12V logic isolation |
| Q1–4 | 4 | N-Channel MOSFET (PWM OUT) | IRLZ44N | TO-220 | 55V, 47A, logic-level gate |
| D1–4 | 4 | Flyback Diode (PWM OUT) | 1N4007 | DO-41 | Inductive load protection |
| D5 | 1 | Reverse Polarity Protection | 1N5819 | DO-41 | Schottky, 40V 1A |
| R1–4 | 4 | 12V Input Resistor (series) | 8.2kΩ 0.5W | 0805 | 12V divider high side |
| R5–8 | 4 | 12V Input Resistor (pull-down) | 3.3kΩ 0.25W | 0805 | 12V divider low side |
| R9–12 | 4 | Optocoupler LED Resistor | 1kΩ 0.25W | 0805 | Limits optocoupler LED current |
| R13–16 | 4 | MOSFET Gate Resistor | 100Ω 0.25W | 0805 | Suppresses gate ringing |
| R17–18 | 2 | Analog Input Voltage Divider | 10kΩ / 3.3kΩ 0.25W | 0805 | For 0–10V analog input range |
| R19–20 | 2 | Button Pull-up (external opt.) | 10kΩ 0.25W | 0805 | Optional — ESP32 has internal pull-ups |
| R21 | 1 | CAN Bus Term. Resistor | 120Ω 0.5W | 0805 | CAN bus termination |
| R22 | 1 | FTDI DTR Auto-Reset | 100nF (cap, see C-section) | — | See C3 |
| C1 | 1 | Bulk Decoupling (12V rail) | 100µF 25V electrolytic | Radial 6.3mm | Input filter |
| C2 | 1 | LDO Output Cap | 10µF 10V electrolytic | Radial 5mm | AMS1117 stability |
| C3 | 1 | FTDI DTR Auto-Reset Cap | 100nF 50V | 0805 | DTR → GPIO0 auto-boot |
| C4 | 1 | GPS VCC Bypass | 10µF 10V electrolytic | Radial 5mm | GPS module supply |
| C5–8 | 4 | MOSFET Gate Bypass | 100nF 50V | 0805 | One per MOSFET |
| J1 | 1 | FTDI Programming Header | 6-pin 2.54mm female | TH | GND/CTS/3V3/TX/RX/DTR |
| J2 | 1 | CAN Bus Connector | 2-pin screw terminal 5mm | TH | CANH / CANL |
| J3 | 1 | PWM Output Connector | 4-pin screw terminal 5mm | TH | OUT1–OUT4 |
| J4 | 1 | 12V Digital Input Connector | 4-pin screw terminal 5mm | TH | DIN1–DIN4 |
| J5 | 1 | Analog Input Connector | 2-pin screw terminal 5mm | TH | AIN1–AIN2 |
| J6 | 1 | Button Connector | 4-pin 2.54mm header | TH | BTN1–BTN4 |
| J7 | 1 | GPS Module Connector | 4-pin 2.54mm header | TH | VCC/GND/TX/RX |
| J8 | 1 | LCD I2C Connector | 4-pin 2.54mm header | TH | VCC/GND/SDA/SCL |
| J9 | 1 | Power Input Connector | 2-pin screw terminal 5mm | TH | 12V / GND |

# 5. Design Notes

## 5.1  12V Digital Inputs

Each 12V digital input is isolated using a PC817 optocoupler. A resistor divider (8.2kΩ series + 3.3kΩ to GND) on the 12V side limits the LED current to approximately 10mA. The collector output is pulled to 3.3V via the ESP32 internal pull-up and drives the GPIO input directly.

## 5.2  PWM Outputs

PWM outputs use the ESP32 LEDC peripheral (up to 16 channels, up to 40 MHz). Each channel drives an IRLZ44N logic-level N-channel MOSFET in a low-side switch configuration. A 100Ω gate resistor suppresses ringing. Add a 1N4007 flyback diode across inductive loads (motors, solenoids). Maximum continuous drain current is 47A per channel — derate appropriately for PCB trace current capacity.

## 5.3  CAN Bus

The ESP32 TWAI controller connects to a SN65HVD230 3.3V-native CAN transceiver. A 120Ω termination resistor is fitted on-board. Use AgIsoStack++ with the ESP-IDF framework for ISOBUS / ISO 11783 protocol support. The TWAI peripheral supports standard and extended frames at up to 1 Mbps.

## 5.4  GPS UART

UART2 is remapped to GPIO 13 (RX) and GPIO 23 (TX) to avoid conflict with PWM outputs on the default UART2 pins. Compatible with u-blox NEO-6M, NEO-8M, and other 3.3V TTL GPS modules at 9600 baud (default). A 10µF decoupling capacitor on the GPS VCC line is recommended to handle acquisition current spikes.

## 5.5  FTDI Programming Header

The 6-pin header is compatible with SparkFun FTDI Basic 3.3V. The DTR pin connects to GPIO0 via a 100nF capacitor and the RTS pin connects to the EN (reset) pin via a 100nF capacitor, implementing the standard Arduino-style auto-reset circuit. This allows one-click flashing from PlatformIO or Arduino IDE without manually pressing the BOOT button.

## 5.6  Power Supply

The board is powered from a 12V supply. An AMS1117-3.3 LDO regulator provides 3.3V for the ESP32 and peripherals. Total 3.3V current draw estimate: ESP32 (~240mA peak), LCD (~20mA), GPS (~50mA), CAN transceiver (~10mA), optocouplers (~5mA) = ~325mA peak. The AMS1117 is rated at 800mA — adequate with margin. Add heatsinking if the 12V–3.3V drop dissipates more than 1W continuously.

NOTE: UART0 (GPIO 1/3) is shared between the FTDI programming header and application serial output. Disconnect the FTDI adapter before connecting any other device to UART0, or use a different UART for application debug output.