Component Suitability Assessment

SISS27DN P-Channel MOSFET as High-Side PWM Output Driver

Triton IO Board  |  Rev 1.0  |  June 2026  |  Author: Design Review

# 1. Verdict

✔ SUITABLE The SISS27DN is suitable as a high-side output driver for the Triton IO Board at up to 10A continuous per channel, when driven by the LTV847 optocoupler with a 1kΩ gate pull-up resistor to the 12V rail. All key electrical parameters are met with adequate margin.

# 2. Component Overview

| Property | SISS27DN | LTV847 |
| --- | --- | --- |
| Manufacturer | Vishay Siliconix | Lite-On Technology |
| Device type | P-Channel MOSFET | Quad phototransistor optocoupler |
| Package | PowerPAK 1212-8S (SMT) | DIP-16 / SOIC-16 |
| Voltage rating | −30V VDS | 35V VCE |
| Current rating | −50A (pkg), −23A (TA=25°C) | 50mA collector |
| Isolation | N/A | 5,000 Vrms |
| Channels | 1 | 4 (one per output channel) |
| Role in circuit | High-side switching element | Logic isolation + gate drive |

# 3. Circuit Topology

Each of the four output channels uses the following topology:

12V Rail

│

│ ┌─────────────────────────────────────────────────┐

├─┤  1kΩ Pull-up Resistor (R_gate)                  │

│ └────────────────┬────────────────────────────────┘

│                  │

│             GATE (SISS27DN)

│             SOURCE ── 12V Rail

│             DRAIN  ── Load (+) terminal

│

│  ┌──────────────────────────────────────────────┐

│  │  LTV847 Channel (one of four)                │

│  │                                              │

│  │  Anode ──── 330Ω ──── ESP32 GPIO (3.3V)     │

│  │  Cathode ── GND                              │

│  │                                              │

│  │  Collector ── GATE (above)                   │

│  │  Emitter  ── GND                             │

│  └──────────────────────────────────────────────┘

│

LOAD

│

GND

Operation:

ESP32 GPIO HIGH → opto LED on → transistor conducts → Gate pulled to GND

→ Vgs = 0 - 12V = -12V → SISS27DN ON → Load energised

ESP32 GPIO LOW  → opto LED off → transistor open → Gate pulled to 12V

→ Vgs = 0V → SISS27DN OFF → Load de-energised (load side at GND)

## 3.1  Gate Drive Mechanism

A 1kΩ resistor connects the 12V rail to the SISS27DN gate. The LTV847 phototransistor collector connects to the gate; the emitter connects to GND. This forms a wired-AND pull-down:

- Opto OFF → gate floats to 12V via 1kΩ → Vgs = GND_ref − 12V = −12V → MOSFET fully enhanced → Output ON

- Opto ON  → gate pulled to GND → Vgs = 0V → MOSFET off → Output OFF → load side sits at GND through the load

## 3.2  Fail-Safe Behaviour

⚠ NOTE The output defaults to ON when the ESP32 is unpowered or the optocoupler LED fails open. Consider whether this fail-safe-ON behaviour is appropriate for connected loads (e.g. solenoid valves, relays). If fail-safe-OFF is required, invert the logic or add a gate pull-down to GND that the opto overrides.

# 4. Electrical Parameter Analysis

## 4.1  SISS27DN Key Parameters

| Parameter | Value (from datasheet) | At 10A load | Verdict |
| --- | --- | --- | --- |
| VDS | −30V max | 12V rail → Vds ≈ −12V | ✔ 2.5× margin |
| ID (TA = 25°C) | −23A continuous | 10A required | ✔ 2.3× margin |
| ID (TA = 70°C) | −18.5A continuous | 10A required | ✔ 1.85× margin |
| Rds(on) @ −10V | 5.6 mΩ max | P = 10² × 0.0056 = 0.56W | ✔ Low conduction loss |
| Rds(on) @ −4.5V | 9.0 mΩ max | P = 10² × 0.009 = 0.90W | ✔ Acceptable |
| VGS(th) | −1.0V to −2.2V | Gate driven to Vgs = −12V | ✔ Fully enhanced |
| VGS max | ±20V | Vgs = −12V in operation | ✔ Well within limit |
| Qg (typical) | ~45 nC | Limits PWM freq (see §4.3) | ⚠ See switching note |
| PD (TA = 25°C) | 4.8W (1"×1" FR4) | 0.56W dissipated | ✔ Ample headroom |
| RthJA (steady) | ~63°C/W | 0.56W × 63 = 35°C rise | ✔ OK with copper pour |

## 4.2  LTV847 Key Parameters

| Parameter | Value | In this circuit | Verdict |
| --- | --- | --- | --- |
| VCE max | 35V | 12V on collector (gate node) | ✔ 2.9× margin |
| IC max | 50mA | 12V / 1kΩ = 12mA when pulling low | ✔ 4.2× margin |
| IF (LED) | 1–60mA | ~10mA with 330Ω from 3.3V GPIO | ✔ Within spec |
| CTR (min) | 50% at IF=5mA | 12mA × 50% = 6mA — more than enough to pull gate low | ✔ |
| VISO | 5,000 Vrms | 12V / 3.3V domain isolation | ✔ Excellent isolation |
| tr / tf | 4µs / 3µs | Limits PWM (see §4.3) | ⚠ See switching note |

## 4.3  Switching Speed and PWM Frequency

The maximum practical PWM frequency is limited by two factors:

- LTV847 rise/fall time: 4µs / 3µs → limits to approximately 1/(10× transition time) ≈ ~25 kHz theoretical

- Gate charge via 1kΩ: t = Qg / I = 45nC / 12mA ≈ 3.75µs turn-on time

Combined, the practical PWM frequency limit is approximately 1–5 kHz. This is suitable for:

  - Proportional valve control

  - Solenoid / relay switching

  - Actuator speed control

  - LED / lamp dimming

⚠ NOTE If PWM frequencies above 5 kHz are required, replace the 1kΩ pull-up with a lower value (e.g. 100Ω) and/or add an active gate driver stage. At higher frequencies, switching losses in the MOSFET will also increase and must be evaluated.

## 4.4  Thermal Analysis

Worst-case conduction loss per channel at 10A continuous:

P_cond = I² × Rds(on) = 10² × 0.009 = 0.90 W  (@ Vgs = −4.5V; actual Vgs = −12V so real Rds(on) ≈ 5.6 mΩ → 0.56 W)

Junction temperature rise above ambient (steady-state, no copper pour):

ΔTj = P × RthJA = 0.56 W × 63°C/W = 35°C

At TA = 70°C: Tj = 70 + 35 = 105°C — within the 150°C TJ max limit.

⚠ PCB THERMAL NOTE The PowerPAK 1212-8S is a small leadless SMT package (3.3 × 3.3 mm) with no heatsink tab. Heat dissipation relies entirely on PCB copper. Provide a minimum 1 cm² copper pour on the drain pad, with thermal vias to inner/bottom layers if using a 4-layer PCB. Without adequate copper, RthJA will be higher than the 63°C/W datasheet figure.

# 5. Supporting Components

| Component | Value | Purpose | Notes |
| --- | --- | --- | --- |
| R_gate (per channel) | 1kΩ 0.25W | Gate pull-up to 12V | Sets Vgs = −12V when opto off; limits IC through opto transistor to 12mA |
| R_led (per channel) | 330Ω 0.125W | LTV847 LED current limiter | (3.3V − 1.2V) / 330Ω ≈ 6.3mA — within CTR spec; increase to 470Ω if noise is a concern |
| D_flyback (per ch.) | 1N4007 / SMAJ | Flyback diode across load | Required for inductive loads (solenoids, motors, relays). Cathode to 12V, anode to drain. |
| C_bypass | 100nF + 10µF | 12V rail decoupling near FETs | Absorbs switching transients; place close to source pins |
| TVS (optional) | SMAJ15A | Drain-source transient clamp | Recommended for industrial environments with long cable runs |

# 6. Comparison with Original Design (IRLZ44N)

| Parameter | IRLZ44N (original) | SISS27DN + LTV847 (new) |
| --- | --- | --- |
| Channel type | N-channel (low-side) | P-channel (high-side) |
| Gate drive | Direct from ESP32 GPIO | Via LTV847 opto + 1kΩ pull-up |
| Logic isolation | None | ✔ 5,000 Vrms via LTV847 |
| Load when OFF | Drain floats / load to 12V | ✔ Load side pulled to GND via load |
| Fail-safe state | OFF (GPIO low) | ON (opto off = gate at 12V) |
| Max continuous ID | 47A @ TC=25°C | 23A @ TA=25°C (board) |
| Rds(on) | 22 mΩ @ Vgs=10V | 5.6 mΩ @ Vgs=−10V |
| Conduction loss @10A | 2.2W | 0.56W (4× lower) |
| Package | TO-220 (through-hole) | PowerPAK 1212-8S (SMT) |
| PWM frequency | Up to ~100 kHz | Practical limit ~1–5 kHz |
| PCB area | Large (TO-220 + heatsink) | ✔ Very small (3.3×3.3mm) |
| BOM cost (est.) | ~€0.80 | ~€0.35 + €0.30 opto = €0.65 |

# 7. Recommendation

✔ APPROVED FOR USE The SISS27DN driven by the LTV847 with a 1kΩ gate pull-up resistor is approved for use in the Triton IO Board output channels at up to 10A continuous and PWM frequencies up to 5 kHz. Verify PCB thermal copper pour and add flyback diodes on all inductive loads.

Required design actions before PCB release:

- Add 1kΩ 0.25W gate pull-up resistor (R_gate) per channel from 12V to MOSFET gate

- Add 330Ω LED current limiter (R_led) per channel on ESP32 GPIO → LTV847 anode path

- Add flyback diode (1N4007 or SMAJ) across each output load

- Provide ≥1 cm² copper pour on SISS27DN drain pad; add thermal vias for 4-layer boards

- Add 100nF + 10µF decoupling on 12V rail near each MOSFET

- Update BOM: replace IRLZ44N with SISS27DN; add LTV847 (1 per board, covers 4 channels)

- Update schematics (EasyEDA JSON and KiCad .kicad_sch) to reflect new topology

# 8. References

Vishay Siliconix — SiSS27DN Datasheet, Document 62847, Rev. A, 13-May-2013

Lite-On Technology — LTV-847 Series Datasheet, High Density Mounting Type Photocoupler

Triton IO Board — Bill of Materials & GPIO Pinout Reference, Rev 1.0, June 2026