# Calibration Procedure

Loofdoes Spuitcomputer  ·  MeijWorks

# 1.  Overview

The Loofdoes Spuitcomputer requires two calibration procedures before it can accurately control the spray dose. Both are performed through the serial monitor (115 200 baud) and the results are stored in non-volatile memory (NVS), so calibration survives power cycles and only needs to be repeated when hardware changes.

| Procedure | Purpose |
| --- | --- |
| 1. Analog input calibration | Maps the dose-setpoint knob (0–4 095 ADC) to the desired application rate in l/ha |
| 2. PWM output calibration | Maps the pump PWM control value (0–1 023) to actual pump flow in ml/min |

# 2.  Prerequisites

USB-serial connection to the ESP32 at 115 200 baud (PlatformIO monitor or any terminal emulator).

All hoses, spray lines, and the pump filled with water and free of air pockets.

A graduated bucket or container for volume measurement.

Machine stationary and in a safe location.

Sufficient water in the tank for all measurement runs (5 × 1 min at maximum flow).

# 3.  Opening the Calibration Menu

Open the serial monitor at 115 200 baud. Press any key to enter calibration mode. The main menu appears:

=== SPRAYER CALIBRATION === 1. Analog input (l/ha) 2. PWM output (ml/min) 3. Show current calibration 4. Edit PWM point 5. Analog output (raw/dose/speed/flow) (OFF - press to toggle) 6. Pump output (calMode/pumpBtn/pumpOn/pumpVal) (OFF - press to toggle) 7. GPS output (OFF - press to toggle) 8. GPS raw passthrough (OFF - press to toggle) q. Exit Choose: _

Type the number of the desired procedure and press Enter. Press q + Enter at any time to exit and return to normal operation.

⚠️  Note: While the calibration menu is open the normal spray dose control loop continues to run in the background. The pump is only driven directly during the PWM output calibration procedure.

# 4.  Analog Input Calibration

This procedure records three reference points that map the physical position of the dose-setpoint knob to an application rate in litres per hectare. The firmware interpolates linearly between the points at runtime.

Type 1 and press Enter.

Set the knob to the fully counter-clockwise (minimum) position and press Enter. The ESP32 reads the raw ADC value and asks for the corresponding dose:

--- Analog point 1/3 --- Set knob to MINIMUM position, then press ENTER. Analog reading: 127 Enter dose for this position (l/ha): _

Enter the target dose for the minimum position (example: 50) and press Enter:

Enter dose for this position (l/ha): 50 Point 1 saved.

Set the knob to the centre (middle) position and press Enter:

--- Analog point 2/3 --- Set knob to MIDDLE position, then press ENTER. Analog reading: 2031 Enter dose for this position (l/ha): _

Enter the target dose for the centre position (example: 100) and press Enter:

Enter dose for this position (l/ha): 100 Point 2 saved.

Set the knob to the fully clockwise (maximum) position and press Enter:

--- Analog point 3/3 --- Set knob to MAXIMUM position, then press ENTER. Analog reading: 3958 Enter dose for this position (l/ha): _

Enter the target dose for the maximum position (example: 200) and press Enter:

Enter dose for this position (l/ha): 200 Point 3 saved. Analog calibration saved.

⚠️  Note: The three knob positions do not need to be set in any particular order; the firmware sorts the calibration points by ADC value automatically.

# 5.  PWM Output Calibration

This procedure characterises the pump flow curve at five equally-spaced PWM control values. For each point the ESP32 runs the pump for exactly one minute; the volume collected in the bucket equals the flow in ml/min.

## 5.1  Finding the Start Threshold

The first step determines the minimum PWM value at which the pump begins to deliver flow. Below this value the pump does not move.

Type 2 and press Enter.

=== PWM OUTPUT CALIBRATION === Turn the analog knob until the pump just starts flowing. Press ENTER to capture that value as the start point.

The pump is now controlled directly by the dose-setpoint knob. Slowly turn the knob clockwise. The current PWM value updates live on the serial monitor:

PWM: 0 PWM: 52 PWM: 147 PWM: 241 PWM: 318 PWM: 342

When the pump just starts delivering a consistent flow, stop and press Enter. The threshold value is captured and the five measurement points are automatically calculated:

Start PWM captured: 342 Point 1/5: PWM = 342. Press ENTER to start 1-minute run.

⚠️  Note: In this example the five points are equally spaced between 342 and 1023: 342 – 512 – 682 – 852 – 1023.

## 5.2  Measuring Flow at Each Point

The ESP32 now steps through the five PWM values one by one. For each point: place the bucket under the outlet and press Enter. The pump runs for exactly 60 seconds with a live countdown, then stops automatically. Measure the collected volume and enter it — the value is the flow in ml/min.

Point 1/5: PWM = 342. Press ENTER to start 1-minute run. 60s remaining 45s remaining 30s remaining 15s remaining 1s remaining Run complete. Enter volume collected (ml): 850 Saved: PWM=342, flow=850 ml/min. Point 2/5: PWM = 512. Press ENTER to start 1-minute run. 60s remaining ... Run complete. Enter volume collected (ml): 1200 Saved: PWM=512, flow=1200 ml/min. Point 3/5: PWM = 682. Press ENTER to start 1-minute run. 60s remaining ... Run complete. Enter volume collected (ml): 1550 Saved: PWM=682, flow=1550 ml/min. Point 4/5: PWM = 852. Press ENTER to start 1-minute run. 60s remaining ... Run complete. Enter volume collected (ml): 1820 Saved: PWM=852, flow=1820 ml/min. Point 5/5: PWM = 1023. Press ENTER to start 1-minute run. 60s remaining ... Run complete. Enter volume collected (ml): 2100 Saved: PWM=1023, flow=2100 ml/min. PWM calibration saved (5 points).

⚠️  Note: If a measurement is incorrect (e.g. the bucket was not ready in time), complete the current session and start a new PWM calibration from option 2. New values overwrite the previous ones in NVS.

# 6.  Editing a PWM Point

Use this option to correct a single PWM calibration point (for example, a bad bucket reading) without repeating the entire five-point PWM run. The corrected value is written to NVS immediately.

Type 4 and press Enter.

=== CURRENT CALIBRATION === ... Select PWM point to edit (1-5, q to cancel): _

Enter the number of the point to edit and press Enter. The current PWM and flow for that point are shown, then enter the corrected flow:

Point 3: PWM=682, flow=1550 ml/min New flow (1-4000 ml/min, q to cancel): 1600 Saved.

⚠️  Note: Enter q instead of a point number, or instead of a flow value, at any point to cancel and return to the menu without changing anything.

# 7.  Live Diagnostic Output

Options 5–8 each toggle a separate live telemetry stream on or off independently. Enabled streams print automatically, once every 500 ms, whenever the calibration menu is idle (i.e. not in the middle of a calibration step) — no further input is needed once a stream is switched on.

## 7.1  Analog Output (Serial Plotter)

Type 5 and press Enter to toggle. Prints the raw analog reading, the interpolated dose, current GPS ground speed, and the resulting pump flow demand, formatted as Arduino Serial Plotter label:value pairs:

raw:2031,dose:100.0,speed:1.85,flow:1550.0

Open Tools ▸ Serial Plotter in the Arduino IDE (or the equivalent view in PlatformIO) at 115 200 baud to graph these four values live while turning the knob or driving — useful for sanity-checking the dose curve in the field without redoing the full calibration.

## 7.2  Pump Output

Type 6 and press Enter to toggle. Prints calibration-mode state, the pump button state, whether the pump output is currently on, whether it’s being PWM-driven, and the raw value sent to the pump:

calMode=N pumpBtn=0 pumpOn=1 pumpPwmFlag=1 pumpVal=682

Not formatted for the Serial Plotter — these are mostly discrete on/off flags, more useful read as text than graphed.

## 7.3  GPS Output

Type 7 and press Enter to toggle. Prints ground speed, position, and the NMEA GGA fix-quality indicator (0 = no fix, 1 = GPS fix, 2 = DGPS fix, etc.):

GPS: speed=1.85 m/s lat=52.123456 lon=5.123456 quality=1

## 7.4  GPS Raw Passthrough

Type 8 and press Enter to toggle. Echoes the raw NMEA sentences received from the GPS receiver directly to the serial monitor — for low-level debugging of the GPS link itself, independent of the parsed output above.

# 8.  Verifying the Calibration

After completing both procedures, select option 3 from the main menu to review the stored values before returning to work.

=== CURRENT CALIBRATION === Analog input (l/ha): 1: analog=127 dose=50 l/ha 2: analog=2031 dose=100 l/ha 3: analog=3958 dose=200 l/ha PWM output (5 points): 1: pwm=342 flow=850 ml/min 2: pwm=512 flow=1200 ml/min 3: pwm=682 flow=1550 ml/min 4: pwm=852 flow=1820 ml/min 5: pwm=1023 flow=2100 ml/min

Verify that the flow values increase monotonically with PWM. If any value is out of order or clearly incorrect, repeat the PWM calibration. Confirm that the dose values match the intended spray prescription for your application.

# 9.  Exiting Calibration

Type q and press Enter from the main menu to exit. The system returns to normal automatic dose control immediately.

Choose: q Exiting calibration.